Name: _______________________________ (   ) Class:______  Date: ______________

Chemistry 5059

Chapter 2: Measurements and Experimental Techniques

Candidates should be able to:

(a) name appropriate apparatus for the measurement of time, temperature, mass and volume,

including burettes, pipettes, measuring cylinders and gas syringes

(b) suggest suitable apparatus, given relevant information, for a variety of simple experiments,

including collection of gases and measurement of rates of reaction.

- Measuring Physical Quantities

Measuring Mass

- SI Unit: kilogram (kg)

- 1 tonne = 1000kg

- Beam balance/ electronic balance

- Accuracy: ±0.01g

Measuring Time

- SI Unit: seconds (s)

- Digital stopwatch

- Accuracy: ±0.01s

Measuring Temperature

- SI Unit: kelvin (K)

- Mercury thermometer/ Alcohol thermometer

- Accuracy: ±0.5℃

Measuring Volume

- SI Unit: cubic metre (m3)

- Measuring Cylinder

  - Accuracy: 0.5 cm3

- Burette

  - Accuracy: 0.05 cm3

- Pipette

  - Fixed volume

- Gas Syringe

  - Measures volume of gas

- Selecting Suitable Apparatus for Experiments

Methods of collecting gases

- Displacement of water

Suitable for collecting gases that are insoluble or slightly soluble in water

- Downward delivery

Suitable for collecting gases that are soluble in water and denser than air

- Upward delivery

Suitable for collecting gases that are soluble in water and less dense than air

| Gas | Solubility in water | Density compared to air | Method of collection |
| --- | --- | --- | --- |
| Hydrogen | Not soluble | Less dense | Displacement of water |
| Oxygen | Very slightly soluble | Slightly denser |  |
| Carbon Dioxide | Slightly soluble | Denser |  |
| Chlorine | Soluble | Denser | Downward delivery |
| Hydrogen Chloride | Very soluble | Denser |  |
| Sulfur Dioxide | Very soluble | Denser |  |
| Ammonia | Extremely soluble | Less dense | Upward delivery |

Drying a gas

- Concentrated sulfuric acid

Used to dry most gases

- Quicklime (calcium oxide)

Used to dry ammonia

- Fused calcium chloride (heated)

Used to dry most gases